# 1: duration of morse code presses

int dotLength = 100

int dashLength = dotLength * 3

int elementPause = dotLength * 3

int spaces = dotLength * 3

int wordPause = dotLength * 7

void setup ()

pinMode (LEDpin, OUTPUT);

void dot ()

digitalWrite (LEDpin, HIGH);

delay (dotLength);

void dash ()

digitalWrite (LEDpin, HIGH);

delay (dashLength);

# 2: connecting button presses to buzzer

const int SW_pin = 2; // digital pin connected to switch output

int dotLength = 100

int dashLength = dotLength * 3

int elementPause = dotLength * 3

int spaces = dotLength * 3

int wordPause = dotLength * 7

int amonG

int buzzerPin = 8;

int switchState = 0;

int prevSwitchState = 0;

unsigned long switchStartTime = 0;

unsigned long switchDuration = 0;

char morse[7] = "";

void setup() {

pinMode(buzzerPin, OUTPUT);

pinMode(SW_pin, INPUT);

digitalWrite(SW_pin, HIGH);

Serial.begin(9600);

}

void loop() {

switchState = (digitalRead(SW_pin));

if (switchState != prevSwitchState) {

if (switchState == HIGH) {

while (switchState == HIGH) {

// while switch is pressed

switchStartTime = millis(); // record start time

digitalWrite(buzzerPin, HIGH);

}

digitalWrite(buzzerPin, LOW);

switchDuration = millis() - switchStartTime;

if (switchDuration < dotLength) {

morse[] += ".";

}

else {

morse[] += "-";

}

return

}

}

if (switchState != HIGH) {

while (switchState != HIGH) {

switchStartTime = millis();

digitalWrite(buzzerPin, LOW);

}

switchDuration = millis() = switchStartTime;

if (switchDuration )

}

}

}

# 3: Joystick with button functionality

const int SW_pin = 2; // digital pin connected to switch output

void setup() {

pinMode(SW_pin, INPUT);

digitalWrite(SW_pin, HIGH);

Serial.begin(9600);

}

void loop() {

Serial.print("Switch:  ");

Serial.print(digitalRead(SW_pin));

Serial.print("\n");

delay(1000);

}

# 4: Beepa Wit Da Joystick 😳

int buzzerPin = 8;

const int SW_pin = 2; // digital pin connected to switch output

void setup() {

pinMode(buzzerPin, OUTPUT);

pinMode(SW_pin, INPUT);

digitalWrite(SW_pin, HIGH);

Serial.begin(9600);

}

void loop() {

int buttonState = digitalRead(SW_pin);

if (buttonState == LOW) {

digitalWrite(buzzerPin, HIGH);

}

if (buttonState == HIGH) {

digitalWrite(buzzerPin, LOW);

}

Serial.print("Switch:  ");

Serial.print(digitalRead(SW_pin));

Serial.print("\n");

}

# 5. LCD Monitor Component Code

(1) Using LCD Displays with Arduino - YouTube

#include <LiquidCrystal.h>

const int rs = 6, e = 7, d4 = 8, d5 = 9, d6 = 10, d7 = 11;

LiquidCrystal lcd(rs, e, d4, d5, d6, d7);

void setup() {

lcd.begin(16,2);

lcd.print ("hello world!");

void loop() {

lcd.setCursor (0,1);

// LCD Monitor Display

#include <LiquidCrystal.h>                                    // include library code

const int rs = 6, e = 7, d4 = 8, d5 = 9, d6 = 10, d7 = 11;    // configure digital pins

LiquidCrystal lcd(rs, e, d4, d5, d6, d7);                     // initialize the library

// LEDpin and Buzzer

int LEDpin = 13;                // assign digital pins to hardware components

int BUZZER = 12;

// Switch

const int SW_pin = 2;

int SWState = 0;                // variable to store the switch state

unsigned long prevTime = 0;     // variable to track the time of switch press

// Morse Code Translation

String morseCode = "";          // declaring variable to store morse code

const int dotDuration = 1000;   // duration of a dot in milliseconds

void setup() {

// set pins to input or output

pinMode(SW_pin, INPUT);

pinMode (LEDpin, OUTPUT);

pinMode (BUZZER, OUTPUT);

// initialize printing on serial monitor with switch

digitalWrite(SW_pin, HIGH);

Serial.begin(9600);

// initialize printing on LCD monitor

lcd.begin(16,2);              // set up columns and rows

}

void loop() {

// LCD Monitor

lcd.setCursor (0,1);                         // set cursor to column 0, line 1

// Reading Switch State

SWState = digitalRead(SW_pin);

Serial.print("Switch:  ");

Serial.print(digitalRead(SW_pin));

Serial.print("\n");

if (SWState == 0) {                          // if switch is pressed

unsigned long currentTime = millis();      // get the current time

digitalWrite(LEDpin, HIGH);                // turn on LED

digitalWrite(BUZZER, HIGH);                // turn on buzzer

// Reading Dots & Dashes in Morse

if (currentTime - prevTime > dotDuration * 3) {

decodeMorse();                           // decode morse code after a pause (word separator)

morseCode = "";                          // reset morse code string

lcd.print ("");

}

else if (currentTime - prevTime > dotDuration) {

morseCode += "-";                        // add dash for a long button press

lcd.print ("-");

}

else {

morseCode += ".";                        // add dot for a short button press

lcd.print (".");

}

prevTime = currentTime;                    // update the previous time

delay(50);                                 // debouncing delay

}

if (SWState == 1) {

digitalWrite(LEDpin, LOW);

digitalWrite (BUZZER, LOW);

}

}

void decodeMorse() {

if (morseCode == ".-") {

Serial.print("A");

} else if (morseCode == "-...") {

Serial.print("B");

} else if (morseCode == "-.-.") {

Serial.print("C");

} else if (morseCode == "-..") {

Serial.print("D");

}

// Add more Morse code translations for other letters and numbers

Serial.print(" "); // Separate characters in the serial monitor

}

# 6. Inserting Morse Code Component into Hardware Components

const int SW_pin = 2; // digital pin connected to switch output

int dotLength = 100;

int dashLength = dotLength * 3;

int elementPause = dotLength * 3;

int spaces = dotLength * 3;

int wordPause = dotLength * 7;

int buzzerPin = 8;

int switchState = 0;

int prevSwitchState = 0;

unsigned long switchStartTime = 0;

unsigned long switchDuration = 0;

unsigned long spaceStartTime = 0;

unsigned long spaceDuration = 0;

String morse = ""; // Using String type for morse code now because idk

String message = ""; // Store the translated message

void setup() {

pinMode(buzzerPin, OUTPUT);

pinMode(SW_pin, INPUT);

digitalWrite(SW_pin, HIGH);

Serial.begin(9600);

}

void loop() {

switchState = digitalRead(SW_pin);

if (switchState != prevSwitchState) {

if (switchState == HIGH) {

switchStartTime = millis(); // record start time

while (switchState == HIGH) { // while switch is pressed

digitalWrite(buzzerPin, HIGH);

switchState = digitalRead(SW_pin); // Update switchState within the loop

}

switchDuration = millis() - switchStartTime;

// Translate button press duration to Morse code??

if (switchDuration < dotLength) {

morse += ".";

} else {

morse += "-";

}

}

if (switchState == LOW) {

spaceStartTime = millis()

while (switchState == LOW) {

digitalWrite(buzzerPin, LOW);

switchState = digitalRead(SW_pin);

}

spaceDuration = millis() - spaceStartTime;

if (spaceDuration > wordPause) {

return

}

}

}

prevSwitchState = switchState; // Update prevSwitchState after checkingthe button state

// Interpret da Morse code and reset for the next character

if (morse.length() > 0) {

char translatedChar = morseToChar(morse);

if (translatedChar != '\0') {

message += translatedChar;

Serial.print("Current Message: ");

Serial.println(message);

}

// Reset morse code string for the next character

morse = "";

}

}

// this is the morseToChar function so we just call on it in the loop above

char morseToChar(String morseCode) {

// Morse code to character mapping

if (morseCode == ".-") return 'A';

else if (morseCode == "-...") return 'B';

else if (morseCode == "-.-.") return 'C';

else if (morseCode == "-..") return 'D';

else if (morseCode == ".") return 'E';

else if (morseCode == "..-.") return 'F';

else if (morseCode == "--.") return 'G';

else if (morseCode == "....") return 'H';

else if (morseCode == "..") return 'I';

else if (morseCode == ".---") return 'J';

else if (morseCode == "-.-") return 'K';

else if (morseCode == ".-..") return 'L';

else if (morseCode == "--") return 'M';

else if (morseCode == "-.") return 'N';

else if (morseCode == "---") return 'O';

else if (morseCode == ".--.") return 'P';

else if (morseCode == "--.-") return 'Q';

else if (morseCode == ".-.") return 'R';

else if (morseCode == "...") return 'S';

else if (morseCode == "-") return 'T';

else if (morseCode == "..-") return 'U';

else if (morseCode == "...-") return 'V';

else if (morseCode == ".--") return 'W';

else if (morseCode == "-..-") return 'X';

else if (morseCode == "-.--") return 'Y';

else if (morseCode == "--..") return 'Z';

else if (morseCode == ".-.-.-") return '.';

else if (morseCode == "--..--") return ',';

else if (morseCode == "..--..") return '?';

else if (morseCode == "-..-.") return '/';

else if (morseCode == ".--.-.") return '@';

else if (morseCode == ".----") return '1';

else if (morseCode == "..---") return '2';

else if (morseCode == "...--") return '3';

else if (morseCode == "....-") return '4';

else if (morseCode == ".....") return '5';

else if (morseCode == "-....") return '6';

else if (morseCode == "--...") return '7';

else if (morseCode == "---..") return '8';

else if (morseCode == "----.") return '9';

else if (morseCode == "-----") return '0';

// If the Morse code is not recognized, return a null character?? idk what this will do to the message but it hsould be nothing

return '\0';

}

Newest Code:

// set variables

const int SW_pin = 2; // sets pin for switch

const int buzzerPin = 6; // sets pin for buzzer

const int buttonPin = 3; // sets pin for button

int LEDPin = 13; // sets LEDPin for built-in LED

int const5VPin = 8; // sets pin 8 as a constant 5V output for the joystick

int dotLength = 500; // time interval to differentiate dots and dashes, also used to determine spaces

int buttonState = 0;

int switchState = 0;

unsigned long switchStartTime = 0;

unsigned long switchDuration = 0;

unsigned long buttonStartTime = 0;

unsigned long buttonDuration = 0;

// declare strings :^D

String morseCode = "";

String message = "";

void setup() {

pinMode(buzzerPin, OUTPUT);

pinMode(LEDPin, OUTPUT);

pinMode(const5VPin, OUTPUT);

pinMode(SW_pin, INPUT);

pinMode(buttonPin, INPUT);

digitalWrite(SW_pin, HIGH);

digitalWrite(const5VPin, HIGH);

Serial.begin(9600);

}

void loop() {

switchState = digitalRead(SW_pin); // start by checking switchState

buttonState = digitalRead(buttonPin);

if (switchState == LOW) {

switchStartTime = millis(); // record start time

digitalWrite(buzzerPin, HIGH);

delay(50);

while (switchState == LOW) {

switchState = digitalRead(SW_pin); // Update switchState within the loop

}

delay(50);

digitalWrite(buzzerPin, LOW);

switchDuration = millis() - switchStartTime;

//Translate button press duration to Morse code??

if (switchDuration < dotLength) {

morseCode += "."; // dot if button was pressed for a shorter time than dotLength

} else {

morseCode += "-";

}

Serial.println(morseCode); //prints current morseCode string

}

// Check for a button press to trigger message translation

buttonState = digitalRead(buttonPin);

switchState = digitalRead(SW_pin);

// if the button is pressed and the switch is not, start translation

if (buttonState == HIGH && switchState == HIGH && morseCode.length() > 0) {

translateMessage (message, morseCode); // translate morseCode into message

buttonStartTime = millis(); // record start time

digitalWrite(LEDPin, HIGH); // turn on built-in LED

delay(50);

while (buttonState == HIGH) { // while switch is pressed

buttonState = digitalRead(buttonPin); // Update buttonState within the loop

switchState = digitalRead(SW_pin);

}

delay(50);

digitalWrite(LEDPin, LOW); // turn off built-in LED

buttonDuration = millis() - buttonStartTime;

if (buttonDuration > dotLength) {

message += "_"; // add a space if button was held down for 1/2 second or more

}

// print the message

Serial.print("Current Message: ");

Serial.println(message);

}

}

// takes message and morseCode as inputs

String translateMessage(String& message, String& morseCode) {

// uses & identifier so the global variable is changed in the function

if (morseCode.length() > 0) {

char translatedChar = morseToChar(morseCode);

morseCode = ""; // resets morseCode for the next character

if (translatedChar != '\0') {

message += translatedChar; // add the translated character to the message

}

}

return message;

}

// generates a character based on the identity of morseCode

char morseToChar(String morseCode) {

// Morse code to character mapping

if (morseCode == ".-") return 'A';

else if (morseCode == "-...") return 'B';

else if (morseCode == "-.-.") return 'C';

else if (morseCode == "-..") return 'D';

else if (morseCode == ".") return 'E';

else if (morseCode == "..-.") return 'F';

else if (morseCode == "--.") return 'G';

else if (morseCode == "....") return 'H';

else if (morseCode == "..") return 'I';

else if (morseCode == ".---") return 'J';

else if (morseCode == "-.-") return 'K';

else if (morseCode == ".-..") return 'L';

else if (morseCode == "--") return 'M';

else if (morseCode == "-.") return 'N';

else if (morseCode == "---") return 'O';

else if (morseCode == ".--.") return 'P';

else if (morseCode == "--.-") return 'Q';

else if (morseCode == ".-.") return 'R';

else if (morseCode == "...") return 'S';

else if (morseCode == "-") return 'T';

else if (morseCode == "..-") return 'U';

else if (morseCode == "...-") return 'V';

else if (morseCode == ".--") return 'W';

else if (morseCode == "-..-") return 'X';

else if (morseCode == "-.--") return 'Y';

else if (morseCode == "--..") return 'Z';

else if (morseCode == ".-.-.-") return '.';

else if (morseCode == "--..--") return ',';

else if (morseCode == "..--..") return '?';

else if (morseCode == "-..-.") return '/';

else if (morseCode == ".--.-.") return '@';

else if (morseCode == ".----") return '1';

else if (morseCode == "..---") return '2';

else if (morseCode == "...--") return '3';

else if (morseCode == "....-") return '4';

else if (morseCode == ".....") return '5';

else if (morseCode == "-....") return '6';

else if (morseCode == "--...") return '7';

else if (morseCode == "---..") return '8';

else if (morseCode == "----.") return '9';

else if (morseCode == "-----") return '0';

// If the Morse code is not recognized, return a null character?? idk what this will do to the message but it hsould be nothing

else {

return '\0';

}

}